Am-ePhoto - Agfa/Olympus/Epson/Sanyo/Nikon digicam support for the Amiga...

Beta release 6.4 - 27th June 1999

===============================================================================

SECTION 1 - SOFTWARE OVERVIEW

===============================================================================

This utility implements the necessary communication routines to allow the

Amiga to talk to digital cameras using the serial protocol adopted by a variety

of manufacturers (Agfa, Olympus, Sanyo, Nikon and Epson).  The specific

functions currently implemented are:

Image download/delete

Read camera ID strings

Beta Requirements

-----------------

# An Amiga with 1MB of free memory (any additional free memory will be

automatically used to buffer downloads)

# A serial port capable of talking at 19200 baud or faster

# A compatible digital camera...

Depending on your serial port connector and the plug fitted to the camera

serial cable, you may also require a 9-pin adapter.

Is my camera compatible?

------------------------

Am-ePhoto is known to work with the following digital cameras.  Olympus cameras

are listed with both their Worldwide and USA-specific model designations.

Afga:    ePhoto307

Epson:   PhotoPC

Olympus: D320L / C840L

D400Z / C900Z

Sanyo:   VPC-G200

Nikon:   Coolpix 900

Coolpix 950

Information currently available suggests that MOST cameras released by these

companies up to the current time (May 1999) use the same serial protocol, and

should therefore work with Am-ePhoto.

Note however that NO guarantees are given, and I cannot be held responsible for

financial loss caused by the purchase of any digital camera not listed above

that is subsequently found not to use the same protocol.  Note in particular

that the Nikon Coolpix 600 is probably NOT compatible...

===============================================================================

SECTION 2 - USING THE SOFTWARE

===============================================================================

Connect the camera to your chosen serial port, following the same procedure

as for connecting to a PC or Mac serial port.  If your serial port has its

own prefs utility, it is recommended that you select RTS/CTS handshaking if

possible. It shouldn't be necessary, but software doesn't always obey the

rules!  Then load Am-ePhoto...

If this is the first time you've used the software, or if you haven't

previously saved your settings, the Am-ePhoto window will open, but no attempt

will be made to talk to the camera.  Fill in the serial comms settings, then

click the Refresh button to initiate comms.  If you want to save your settings

for future use, click the (wait for it) Save Prefs button...  Once saved, your

settings will be automatically loaded each time you come to start Am-ePhoto

in future.

Baud Rate (19200, 38400, 57600, 115200, 230400)

By default the camera talks to the host at 19k2 bits/sec, but it supports

speeds from 19k2 up to 230k4.  This option allows you to *attempt*

communication at any of these speeds.  But note the emphasis on attempt -

there is no guarantee that your serial port will reliably support speeds

higher than 19k2 or 38k4.  The highest speed attainable seems to depend

on your serial hardware as well as the camera type... my ePhoto307 can only

manage 38400 (even though it works OK on a PC using 115200 - wierd), whereas

using the same serial port, my C900Z is quite happy at 115200.

At the time of writing, 230k4 is untested - even if both your camera and

serial port support this speed, it may still fail...

Std Delay (1-100)

This is the delay, in 50ths of a second, to wait between checks of the serial

port for incoming data.  My ePhoto307 works reliably with a setting of 1,

whereas my C900Z prefers a setting of 10.  Please experiment with your camera

and report the lowest delay setting that works reliably for you, along with

your camera type and Amiga details.

Feedback Delay (50-200)

This is the delay, in 50ths of a second, to wait between checks of the serial

port FOR FUNCTIONS THAT DO NOT RETURN A VALUE IMMEDIATELY.  At the moment,

the only such function in this version of Am-ePhoto is Delete All.  If this

appears to fail even though it has actually operated correctly according

to the readout on the camera itself, increase this delay value.

For the ePhoto307 and C900Z (with a 4MB flash card), a value of 50 appears

to be reliable.  Larger flash cards take longer to delete, and may therefore

require a higher delay setting, although the delete function does try to

take this into account when waiting for the camera to respond.

Device

Select whichever serial device is appropriate for the serial port hardware

you'd like to use.  If you're using the standard serial port, rather than

a 3rd party add-on, then serial.device should work just fine.

MAKE SURE you spell the device name correctly!!!  The Guru loves typos :-)

Unit

Select the appropriate unit for the serial port in question.  For the default

serial port, a setting of 0 is required.

If you change any of these settings, or if you connect a different camera

whilst Am-ePhoto is still running, you must then tell Am-ePhoto to reconnect

to the camera using the new settings.  To do so, click Refresh...

The Download All and Erase All buttons do exactly what they say...  To select

the base path/filename for downloaded images, enter the appropriate string

into the "Image path" gadget, and then HIT THE RETURN KEY!  Remember that

Am-ePhoto automatically tags the photo number and .jpg extension onto the end

of this filename (even if the photo is actually a TIFF...  OK, so that's

something to change in a future beta, just live with it for now :-)

e.g.

To save all the images to an "images" directory on hd2:, using "Img-" as

the base filename:

Image path = hd2:images/Img-

Am-ePhoto will then save the images as hd2:images/Img-0001.jpg,

hd2:images/Img-0002.jpg etc.

Whilst downloading, a "Download Progress" window will open, containing a

progress bar and two Cancel gadgets.  The first, "Cancel This", aborts

the current image download, but will continue downloading any remaining

images in the camera.  The second, "Cancel All", aborts ALL remaining

image downloads.

===============================================================================

SECTION 3 - DEVELOPER NOTES

===============================================================================

This utility has been developed by Chris Coulson using AMOS Pro/Compiler with

the LSerial and OS DevKit extensions running on an A4000/060.  Camera serial

comms protocol information was obtained using a Win95 PII-300 running the Agfa

and Olympus camera utilities and Comlite32, a most excellent Win95 serial port

sniffer utility.  Additional protocol details were obtained via online sources.

Comments, suggestions and bug reports should be sent to

ephoto@tornado.pair.com

NOTE:  If this archive was downloaded from anywhere OTHER than the Am-ePhoto

support site (http://www.tornado.pair.com/amiga/ephoto.html), it may not be

a valid archive, and is probably not the most recent (i.e. least buggy)

version available...  Until Am-ePhoto is ready for a full public release,

please use the support site to obtain new versions, and notify me about any

other sites that are distributing Am-ePhoto archives.

Thanks for testing!

Chris